P.O. Box 2892

Delmas, 2210

Mpumalanga

Telephone: 013 668 0000

Facsimile: 086 659 8207

Email: info@premac.co.za

VAT No: 4810183410

www.premac.co.za

Manufacturers of Hydraulic Equipment

VALVE RANGE

The locally manufactured V75, V150, V200 and V260 stack type directional control valves are manufactured for the South African industrial, agricultural and mobile markets, offering an extensive range of combinations and options to suit every requirement.

Premac manufactures stack-type directional control valves that are equivalent to Gresen/Parker and Commercial valves. Premac has been manufacturing these valves locally in South Africa since 1989 and has proven its reliability, offering an extensive range of combinations and options to suit every requirement.

The range offers several safety features such as built-in main relief, anti-cavitational checks, load holding checks and port relief cartridges. These valves are manufactured to the highest specifications using the best material available to ensure a lasting good quality product.

Premac’s range offers valves with capacity’s ranging from 75 litres per minute through to 260 litres per minute. Each valves’ specifications are detailed below.

V75 VALVE RANGE

SPECIFICATIONS:

Capacity:	75 Litres/min

Working pressure: 	240 Bar

Number of spools available:	9 sections

Spool options:		3 Way 3 Position

4 Way 3 Position

4 Way 4 Position Float

4 Way 3 Position Free Flow (Motor)

Actuator:	Spring centered to neutral

3 Position Detent

2 Position Detent

Hydraulic Remote

Main Relief:	Differential Pressure Poppet

Miscellaneous:	Power Beyond

Open Centre

Closed Centre

Work Port Relief

Work Port Anti-Cavitation Check

Mid-inlet

Inlet Cover		             Work Section			         Outlet Cover

SAE12					SAE10					SAE12

V150 VALVE RANGE

SPECIFICATIONS:

Capacity:	150 Litres/min

Working pressure: 	210 Bar

Number of spools available:	9 sections

Spool options:		3 Way 3 Position

4 Way 3 Position

4 Way 4 Position Float

4 Way 4 Position Regenerative

4 Way 3 Position Free Flow (Motor)

Actuator:	Spring centered to neutral

3 Position Detent

2 Position Detent

Hydraulic Remote

Main Relief:	Differential Pressure Poppet

Miscellaneous:	Power Beyond

Open Centre

Closed Centre

Conversion Plug

Load Holding Check

Work Port Relief

Work Port Anti-Cavitation Check

Inlet Cover		                   Work Section			         Outlet Cover

SAE 16 or 20 	   		             SAE 12 or 16				SAE 20

V200 VALVE RANGE

SPECIFICATIONS:

Capacity:	200 Litres/min

Working pressure: 	210 Bar

Number of spools available:	8 sections

Actuator:	Spring centered to neutral

3 Position Detent

2 Position Detent

Hydraulic Remote

Main Relief:	Differential Pressure Poppet

Miscellaneous:	Power Beyond

Open Centre

Closed Centre

Conversion Plug

Load Holding Check

Work Port Relief

Work Port Anti-Cavitation Check

Inlet Cover		               Work Section			         Outlet Cover

SAE 20			              SAE 16					SAE 20