Agrigel

# Control Area Network Bus Configuration

| Version: | 2018.07.25.1 | Date: | 25/07/2018 |
| --- | --- | --- | --- |
| Document: | AGR001 | Author: | Jacques Duvenage |

# CAN Bus properties

Speed: 250kbit/s

## Sampling Setup

High Level Protocol: CAN Kingdom

# Program Logic

# CAN Kingdom Implementation

## Design

Flow Control Units have their row number saved in EEPROM and the unit loads this value on startup

## Standard Documents

| Document | Envelope |
| --- | --- |
| King’s Document | 0 |
| Mayor’s Document | 1 |
| Configuration Document | 3 |
| Base Number | 10 |
| Device Addresses | 11-265 |
| Global Flow Control Capital Transmit Envelope | 300 |
| Envelopes that can be assigned to folders | 301-3000 |

### King’s Document

The King’s document contains the first three pages. The envelope assignment is implemented as follows:

### Mayor’s Document

The mayor’s document implements the first two pages only.  The system we are using will only use the Serial Code to identify cities

### Configuration Document

This document helps set up the different devices in the system for their own specific purpose. It helps to identify devices that are not initialized and is also used to assign and receive ID numbers.

### Flow Control Messages

This document facilitates flow control information exchange. Units are engaged and disengaged or their flow rate is changed with this document. One specific envelope is assigned to all units as the global flow control envelope.

### FC Peripheral Control Document

This document facilitates the setting up and control of all other peripheral units in the system.

## Forms, Lists and Documents

### Capital

| List | Name |
| --- | --- |
| T1 | Main Transmit List |
| R1 | Main Receive List |

| Document | Number | List |
| --- | --- | --- |
| King’s Document (Transmit) | 0 | T1 |
| Configuration Document (Transmit) | 1 | T1 |
| Flow Control Document (Transmit) | 2 | T1 |
| FC Peripheral Control (Transmit) | 3 | T1 |
| Mayor’s Document (Receive) | 0 | R1 |
| Configuration Document (Receive) | 1 | R1 |
| Flow Control Document (Receive) | 2 | R1 |
| FC Peripheral Control (Receive) | 3 | R1 |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page 0. Terminates the setup phase. Orders a Mayor to set his City into a specific working mode, e.g. in a Run or Freeze mode. |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000000 | 0 |  | Page zero |
| 2 | rrrrrrAA | AA = 00 |  | Keep current mode |
|  |  | AA = 01 |  | Run |
|  |  | AA = 10 |  | Freeze |
|  |  | AA = 11 |  | Reset |
|  |  | R = reserved |  |  |
| 3 | rrrrrrCC | CC = 00 |  | Keep current mode |
|  |  | CC = 01 |  | Silent |
|  |  | CC = 10 |  | Listen Only |
|  |  | CC = 11 |  | Communicate |
|  |  | R = reserved |  |  |
| 4 | MMMMMMMM | M = 0 |  | Keep Current Mode |
|  |  | M = anything else |  | Modes according to city specs |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page 1. Initiating Page. Provides the Base Number and task for mayor’s response |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000001 | 1 |  | Page one |
| 2 | xxxxxxxx | 0xFF |  | No response requested |
|  |  | Anything else |  | The mayor’s respone Page in the Mayor’s Document |
| 3 | Rrrrrrrr | R = 0 |  | Reserved |
| 4 | Xxxxxxxx | Base number, LSB |  |  |
| 5 | Xxxxxxxx | Base Number |  |  |
| 6 | Xxxxxxxx | Base Number |  |  |
| 7 | Errxxxxx | Base number, MSB |  |  |
|  |  | All x = 0 |  | Keep current setting |
|  |  | All x = 1 |  | Base number undefined |
|  |  | r = reserved |  |  |
|  |  | E = 1 |  | Extended format |
|  |  | E = 0 |  | Standard format |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 2 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page2. Assigning an envelope to or expelling an envelope from a folder forming letters and restraining the use of an envelope |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000010 | 2 |  | Page two |
| 2 | xxxxxxxx | Envelope number, LSB |  |  |
| 3 | Xxxxxxxx | Envelope number |  |  |
| 4 | Xxxxxxxx | Envelope number |  |  |
| 5 | ECrxxxxx | Envelope number, MSB |  |  |
|  |  | R = reserved |  |  |
|  |  | C = 1 |  | Compressed envelope |
|  |  | C = 0 |  | Normal envelope |
|  |  | E = 1 |  | Extended format |
|  |  | E = 0 |  | Standard format |
| 6 | Xxxxxxxx | Folder Number |  |  |
| 7 | rrrrrAAE | E = 1 |  | Enable Folder |
|  |  | E = 0 |  | Disable folder |
|  |  | AA =00 |  | Keep current assignment |
|  |  | AA = 01 |  | Assign this envelope to the Folder on Line 6 and keep all other previous assignments in parallel. |
|  |  | A = 11 |  | Transfer the current assignment of this envelope to the Folder on Line 6. |
|  |  | AAE = 100 |  | Expel this envelope from any assignment |
|  |  | Rrrrr = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Mayor’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The Mayor’s Page 0. City Identification, EAN-13 Code |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned 40-bit integer, LSB |  | EAN-13 Code |
| 3 | xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 4 | Xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 5 | Xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 6 | Xxxxxxxx | Unsigned 40-bit integer, MSB |  | EAN-13 Code |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Mayor’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The Mayor’s Page 1. City Identification, Serial Number |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | xxxxxxxx | Unsigned 40-bit integer, LSB |  | Serial Number |
| 3 | xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 4 | Xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 5 | Xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 6 | Xxxxxxxx | Unsigned 40-bit integer, MSB |  | Serial Number |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 0. Notify all devices to reply with non-init status warning if device does not have an assigned device ID |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | xxxxxxxx | 0:255 |  | Device ID |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | CC000D0A |  |  | A: Request non-init devices to respond (0: Do nothing , 1: Please respond) D: Request config details from deivce indicated in Line 0 (Device ID) C: Enter Config Mode (11: Enter Config, 0x: Do nothing, 10: Do nothing) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 1. Indicates configuration status of device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | xxxxxxxx | 0:255 |  | Device ID (if Initialised) |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | 000000PA |  |  | A: Initialisation Status (0: Not Init , 1: Init) P: Push Button Init Received (0: Nothing Happened, 1: Button was pushed) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 2 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 2. Sends a new device id to unit |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000010 | 2 |  | Page 2 |
| 2 | xxxxxxxx | Unsigned 8-bit integer |  | New device ID (Range = 0:255) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Flow Control Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 2 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Flow Control document. Page 0. Receive the current flow rate of a device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned16-bit integer, LSB |  | Flow Rate |
| 3 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Flow Rate |
| 4 | 000000ER | Bit Flags |  | E: Engaged Status (0: No, 1: Yes) R: Error Status (0: Working, 1: Error) |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | Flow Control Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 2 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Flow Control document. Page 1. Change the current flow rate of a device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | xxxxxxxx | Unsigned16-bit integer, LSB |  | Flow Rate |
| 3 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Flow Rate |
| 4 | 000RAUE0 | Bit Flags |  | E: Engage (0: No, 1: Yes) U: Update Flow Rate (0: No, 1: Yes) A: Update Engage (0: No, 1: Yes) R: Reply with current state of flow rate (0: No, 1: Yes) |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Main Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | Capital |  |
| Code: |  |  | None |  |
| Document Name: |  |  | FC Peripheral Control Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 3 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Peripheral Control document. Page 0. Receive the current peripheral state of the device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned 16-bit integer, LSB |  | Pressure (100 = 1Bar) |
| 3 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Pressure (100 = 1Bar) |
| 4 | 0000SPER | Bit Flags |  | E: Engaged Status (0: No, 1: Yes) R: Error Status (0: Functioning Properly, 1: Error) P:Pickup Speed Update(0: No, 1:Yes) S:Pressure Update(0: No, 1:Yes) |
| 5 | xxxxxxxx | Unsigned 16-bit integer, LSB |  | Current Pickup Pulse (ms) |
| 6 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Current Pickup Pulse (ms) |
| 7 | Rrrrrrrr | R = reserved |  |  |

### Flow Control Unit

| List | Name |
| --- | --- |
| T1 | Main Transmit List |
| R1 | Main Receive List |

| Document | Number | List | Envelope |
| --- | --- | --- | --- |
| Mayor’s Document (Transmit) | 0 | T1 | 0 |
| Configuration Document (Transmit) | 1 | T1 | 2 |
| Flow Control Document (Transmit) | 2 | T1 | 4 |
| King’s Document (Receive) | 0 | R1 | 1 |
| Configuration Document (Transmit) | 1 | R1 | 3 |
| Flow Control Document (Receive) | 2 | R1 | 5 |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page 0. Terminates the setup phase. Orders a Mayor to set his City into a specific working mode, e.g. in a Run or Freeze mode. |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000000 | 0 |  | Page zero |
| 2 | rrrrrrAA | AA = 00 |  | Keep current mode |
|  |  | AA = 01 |  | Run |
|  |  | AA = 10 |  | Freeze |
|  |  | AA = 11 |  | Reset |
|  |  | R = reserved |  |  |
| 3 | rrrrrrCC | CC = 00 |  | Keep current mode |
|  |  | CC = 01 |  | Silent |
|  |  | CC = 10 |  | Listen Only |
|  |  | CC = 11 |  | Communicate |
|  |  | R = reserved |  |  |
| 4 | MMMMMMMM | M = 0 |  | Keep Current Mode |
|  |  | M = anything else |  | Modes according to city specs |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page 1. Initiating Page. Provides the Base Number and task for mayor’s response |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000001 | 1 |  | Page one |
| 2 | xxxxxxxx | 0xFF |  | No response requested |
|  |  | Anything else |  | The mayor’s response Page in the Mayor’s Document |
| 3 | Rrrrrrrr | R = 0 |  | Reserved |
| 4 | Xxxxxxxx | Base number, LSB |  |  |
| 5 | Xxxxxxxx | Base Number |  |  |
| 6 | Xxxxxxxx | Base Number |  |  |
| 7 | Errxxxxx | Base number, MSB |  |  |
|  |  | All x = 0 |  | Keep current setting |
|  |  | All x = 1 |  | Base number redefined |
|  |  | Rr = reserved |  |  |
|  |  | E = 1 |  | Extended format |
|  |  | E = 0 |  | Standard format |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 2 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page2. Assigning an envelope to or expelling an envelope from a folder forming letters and restraining the use of an envelope |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000010 | 2 |  | Page two |
| 2 | xxxxxxxx | Envelope number, LSB |  |  |
| 3 | xxxxxxxx | Envelope number |  |  |
| 4 | Xxxxxxxx | Envelope number |  |  |
| 5 | ECrxxxxx | Envelope number, MSB |  |  |
|  |  | R = reserved |  |  |
|  |  | C = 1 |  | Compressed envelope |
|  |  | C = 0 |  | Normal envelope |
|  |  | E = 1 |  | Extended format |
|  |  | E = 0 |  | Standard format |
| 6 | Xxxxxxxx | Folder Number |  |  |
| 7 | rrrrrAAE | E = 1 |  | Enable Folder |
|  |  | E = 0 |  | Disable folder |
|  |  | AA =00 |  | Keep current assignment |
|  |  | AA = 01 |  | Assign this envelope to the Folder on Line 6 and keep all other previous assignments in parallel. |
|  |  | A = 11 |  | Transfer the current assignment of this envelope to the Folder on Line 6. |
|  |  | AAE = 100 |  | Expel this envelope from any assignment |
|  |  | Rrrrr = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Mayor’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The Mayor’s Page 0. City Identification, EAN-13 Code |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned 40-bit integer, LSB |  | EAN-13 Code |
| 3 | xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 4 | Xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 5 | Xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 6 | Xxxxxxxx | Unsigned 40-bit integer, MSB |  | EAN-13 Code |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Mayor’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The Mayor’s Page 1. City Identification, Serial Number |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | xxxxxxxx | Unsigned 40-bit integer, LSB |  | Serial Number |
| 3 | xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 4 | Xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 5 | Xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 6 | Xxxxxxxx | Unsigned 40-bit integer, MSB |  | Serial Number |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 0. Notify all devices to reply with non-init status warning if device does not have an assigned device ID |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | CC00000A |  |  | A: Request non-init devices to respond (0: Do nothing , 1: Please respond) C: Enter Config Mode (11: Enter Config, 0x: Do nothing, 10: Do nothing) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 1. Indicates configuration status of device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | 000000PA |  |  | A: Initialisation Status (0: Not Init , 1: Init) P: Push Button Init Received (0: Nothing Happened, 1: Button was pushed) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 2 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 2. Sends a new device id to unit |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000010 | 2 |  | Page 2 |
| 2 | xxxxxxxx | Unsigned 8-bit integer |  | New device ID (Range = 0:255) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Flow Control Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Flow Control document. Page 0. Transmit the current flow rate of the device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned16-bit integer, LSB |  | Flow Rate |
| 3 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Flow Rate |
| 4 | 000000ER | Bit Flags |  | E: Engaged Status (0: No, 1: Yes) R: Error Status (0: Working, 1: Error) |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Flow Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xA500010000 |  |
| Document Name: |  |  | Flow Control Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Flow Control document. Page 1. Change the current flow rate of the device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | xxxxxxxx | Unsigned16-bit integer, LSB |  | Flow Rate |
| 3 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Flow Rate |
| 4 | 000RAUE0 | Bit Flags |  | E: Engage (0: No, 1: Yes) U: Update Flow Rate (0: No, 1: Yes) A: Update Engage (0: No, 1: Yes) R: Reply with current state of flow rate (0: No, 1: Yes) |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

## FC Peripheral Unit

### Flow Control Unit

| List | Name |
| --- | --- |
| T1 | Main Transmit List |
| R1 | Main Receive List |

| Document | Number | List | Envelope |
| --- | --- | --- | --- |
| Mayor’s Document (Transmit) | 0 | T1 | 0 |
| Configuration Document (Transmit) | 1 | T1 | 2 |
| Peripheral Control Document (Transmit) | 3 | T1 | 4 |
| King’s Document (Receive) | 0 | R1 | 1 |
| Configuration Document (Transmit) | 1 | R1 | 3 |
| Peripheral Control Document (Receive) | 3 | R1 | 5 |

| Name: |  |  | Peripheral Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page 0. Terminates the setup phase. Orders a Mayor to set his City into a specific working mode, e.g. in a Run or Freeze mode. |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000000 | 0 |  | Page zero |
| 2 | rrrrrrAA | AA = 00 |  | Keep current mode |
|  |  | AA = 01 |  | Run |
|  |  | AA = 10 |  | Freeze |
|  |  | AA = 11 |  | Reset |
|  |  | R = reserved |  |  |
| 3 | rrrrrrCC | CC = 00 |  | Keep current mode |
|  |  | CC = 01 |  | Silent |
|  |  | CC = 10 |  | Listen Only |
|  |  | CC = 11 |  | Communicate |
|  |  | R = reserved |  |  |
| 4 | MMMMMMMM | M = 0 |  | Keep Current Mode |
|  |  | M = anything else |  | Modes according to city specs |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Peripheral Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page 1. Initiating Page. Provides the Base Number and task for mayor’s response |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000001 | 1 |  | Page one |
| 2 | xxxxxxxx | 0xFF |  | No response requested |
|  |  | Anything else |  | The mayor’s response Page in the Mayor’s Document |
| 3 | Rrrrrrrr | R = 0 |  | Reserved |
| 4 | Xxxxxxxx | Base number, LSB |  |  |
| 5 | Xxxxxxxx | Base Number |  |  |
| 6 | Xxxxxxxx | Base Number |  |  |
| 7 | Errxxxxx | Base number, MSB |  |  |
|  |  | All x = 0 |  | Keep current setting |
|  |  | All x = 1 |  | Base number redefined |
|  |  | Rr = reserved |  |  |
|  |  | E = 1 |  | Extended format |
|  |  | E = 0 |  | Standard format |

| Name: |  |  | Peripheral Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | King’s Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 2 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The King’s Page2. Assigning an envelope to or expelling an envelope from a folder forming letters and restraining the use of an envelope |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Xxxxxxxx | 0x00 – 0xFF |  | City or Group address |
| 1 | 00000010 | 2 |  | Page two |
| 2 | xxxxxxxx | Envelope number, LSB |  |  |
| 3 | xxxxxxxx | Envelope number |  |  |
| 4 | Xxxxxxxx | Envelope number |  |  |
| 5 | ECrxxxxx | Envelope number, MSB |  |  |
|  |  | R = reserved |  |  |
|  |  | C = 1 |  | Compressed envelope |
|  |  | C = 0 |  | Normal envelope |
|  |  | E = 1 |  | Extended format |
|  |  | E = 0 |  | Standard format |
| 6 | Xxxxxxxx | Folder Number |  |  |
| 7 | rrrrrAAE | E = 1 |  | Enable Folder |
|  |  | E = 0 |  | Disable folder |
|  |  | AA =00 |  | Keep current assignment |
|  |  | AA = 01 |  | Assign this envelope to the Folder on Line 6 and keep all other previous assignments in parallel. |
|  |  | A = 11 |  | Transfer the current assignment of this envelope to the Folder on Line 6. |
|  |  | AAE = 100 |  | Expel this envelope from any assignment |
|  |  | Rrrrr = reserved |  |  |

| Name: |  |  | Peripheral Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | Mayor’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The Mayor’s Page 0. City Identification, EAN-13 Code |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned 40-bit integer, LSB |  | EAN-13 Code |
| 3 | xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 4 | Xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 5 | Xxxxxxxx | Unsigned 40-bit integer |  | EAN-13 Code |
| 6 | Xxxxxxxx | Unsigned 40-bit integer, MSB |  | EAN-13 Code |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Peripheral Control |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | Mayor’s Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 0 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| The Mayor’s Page 1. City Identification, Serial Number |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | xxxxxxxx | Unsigned 40-bit integer, LSB |  | Serial Number |
| 3 | xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 4 | Xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 5 | Xxxxxxxx | Unsigned 40-bit integer |  | Serial Number |
| 6 | Xxxxxxxx | Unsigned 40-bit integer, MSB |  | Serial Number |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Peripheral Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 0. Notify all devices to reply with non-init status warning if device does not have an assigned device ID |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | CC00000A |  |  | A: Request non-init devices to respond (0: Do nothing , 1: Please respond) C: Enter Config Mode (11: Enter Config, 0x: Do nothing, 10: Do nothing) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Peripheral Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 1 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 1. Indicates configuration status of device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000001 | 1 |  | Page 1 |
| 2 | 000000PA |  |  | A: Initialisation Status (0: Not Init , 1: Init) P: Push Button Init Received (0: Nothing Happened, 1: Button was pushed) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Peripheral Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | Configuration Document |  |
| Document List: |  |  | R1 |  |
| Document Number: |  |  | 1 |  |
| Document Type: |  |  | Receive |  |
| Page Number: |  |  | 2 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Configuration Document. Page 2. Sends a new device id to unit |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000010 | 2 |  | Page 2 |
| 2 | xxxxxxxx | Unsigned 8-bit integer |  | New device ID (Range = 0:255) |
| 3 | Rrrrrrrr | R = reserved |  |  |
| 4 | Rrrrrrrr | R = reserved |  |  |
| 5 | Rrrrrrrr | R = reserved |  |  |
| 6 | Rrrrrrrr | R = reserved |  |  |
| 7 | Rrrrrrrr | R = reserved |  |  |

| Name: |  |  | Peripheral Control Unit |  |
| --- | --- | --- | --- | --- |
| City Type: |  |  | City |  |
| Code: |  |  | 0xFC00010000 |  |
| Document Name: |  |  | FC Peripheral Control Document |  |
| Document List: |  |  | T1 |  |
| Document Number: |  |  | 3 |  |
| Document Type: |  |  | Transmit |  |
| Page Number: |  |  | 0 |  |
| Number of Lines: |  |  | 8 |  |
| Data description: |  |  |  |  |
| Peripheral Control document. Page 0. Transmit the current peripheral of the device |  |  |  |  |
| Line | Format | Values |  | Description |
| 0 | Rrrrrrrr | R = reserved |  |  |
| 1 | 00000000 | 0 |  | Page 0 |
| 2 | xxxxxxxx | Unsigned 16-bit integer, LSB |  | Pressure (100 = 1Bar) |
| 3 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Pressure (100 = 1Bar) |
| 4 | 0000SPER | Bit Flags |  | E: Engaged Status (0: No, 1: Yes) R: Error Status (0: Functioning Properly, 1: Error) P:Pickup Speed Update(0: No, 1:Yes) S:Pressure Update(0: No, 1:Yes) |
| 5 | xxxxxxxx | Unsigned 16-bit integer, LSB |  | Current Pickup Pulse (ms) |
| 6 | xxxxxxxx | Unsigned 16-bit integer, MSB |  | Current Pickup Pulse (ms) |
| 7 | Rrrrrrrr | R = reserved |  |  |