# EEPROM Configuration

|  | Register |  | Register |  | Register |  | Register |  | Register |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0000 | SMSTH | 0x002D |  | 0x005A | LGADL | 0x0087 |  | 0x00B4 | DRXL0 |
| 0x0001 | SMSTL | 0x002E |  | 0x005B |  | 0x0088 | GDY1 | 0x00B5 | DTYP0 |
| 0x0002 | SYSSRC | 0x002F |  | 0x005C |  | 0x0089 | GDY0 | 0x00B6 | DVID1 |
| 0x0003 | SERLH | 0x0030 | FRTDH | 0x005D |  | 0x008A | GDM1 | 0x00B7 | DTXH1 |
| 0x0004 | SERLL | 0x0031 | FRTDL | 0x005E |  | 0x008B | GDM0 | 0x00B8 | DTXL1 |
| 0x0005 | TMZN | 0x0032 | FERTTP | 0x005F |  | 0x008C | GDD1 | 0x00B9 | DRXH1 |
| 0x0006 |  | 0x0033 |  | 0x0060 | LRDS | 0x008D | GDD0 | 0x00BA | DRXL1 |
| 0x0007 |  | 0x0034 |  | 0x0061 | LRDM | 0x008E |  | 0x00BB | DTYP1 |
| 0x0008 |  | 0x0035 |  | 0x0062 | LRDH | 0x008F |  | 0x00BC | DVID2 |
| 0x0009 |  | 0x0036 |  | 0x0063 |  | 0x0090 | IMPLTP | 0x00BD | DTXH2 |
| 0x000A |  | 0x0037 |  | 0x0064 |  | 0x0091 |  | 0x00BE | DTXL2 |
| 0x000B |  | 0x0038 |  | 0x0065 |  | 0x0092 | IMPLID | 0x00BF | DRXH2 |
| 0x000C |  | 0x0039 |  | 0x0066 |  | 0x0093 |  | 0x00C0 | DRXL2 |
| 0x000D |  | 0x003A |  | 0x0067 | LRDVO | 0x0094 |  | 0x00C1 | DTYP2 |
| 0x000E |  | 0x003B |  | 0x0068 |  | 0x0095 |  | 0x00C2 | DVID3 |
| 0x000F |  | 0x003C |  | 0x0069 |  | 0x0096 |  | 0x00C3 | DTXH3 |
| 0x0010 | TNKCH | 0x003D |  | 0x006A |  | 0x0097 |  | 0x00C4 | DTXL3 |
| 0x0011 | TNKCL | 0x003E |  | 0x006B | LRDWE | 0x0098 |  | 0x00C5 | DRXH3 |
| 0x0012 | TNKIH | 0x003F |  | 0x006C |  | 0x0099 |  | 0x00C6 | DRXL3 |
| 0x0013 | TNKIL | 0x0040 |  | 0x006D | LRDHE | 0x009A |  | 0x00C7 | DTYP3 |
| 0x0014 | TNKLH | 0x0041 |  | 0x006E |  | 0x009B |  | 0x00C8 | DVID4 |
| 0x0015 | TNKLL | 0x0042 | SEEDTP | 0x006F |  | 0x009C | ROWS | 0x00C9 | DTXH4 |
| 0x0016 | TNKMH | 0x0043 |  | 0x0070 | LRGS | 0x009D | RWWD | 0x00CA | DTXL4 |
| 0x0017 | TNKML | 0x0044 |  | 0x0071 | LRGM | 0x009E | SIMSPD | 0x00CB | DRXH4 |
| 0x0018 | TNKAH | 0x0045 |  | 0x0072 | LRGH | 0x009F |  | 0x00CC | DRXL4 |
| 0x0019 | TNKAL | 0x0046 |  | 0x0073 |  | 0x00A0 | RGDEV | 0x00CD | DTYP4 |
| 0x001A | TNPMH | 0x0047 |  | 0x0074 |  | 0x00A1 | FDEV | 0x00CE | DVID5 |
| 0x001B | TNPML | 0x0048 |  | 0x0075 |  | 0x00A2 | PDEV | 0x00CF | DTXH5 |
| 0x001C | TNPIH | 0x0049 |  | 0x0076 |  | 0x00A3 | ENVFH | 0x00D0 | DTXL5 |
| 0x001D | TNPIL | 0x004A |  | 0x0077 | LRGVO | 0x00A4 | ENVFL | 0x00D1 | DRXH5 |
| 0x001E |  | 0x004B |  | 0x0078 |  | 0x00A5 | DEVID | 0x00D2 | DRXL5 |
| 0x001F |  | 0x004C |  | 0x0079 |  | 0x00A6 |  | 0x00D3 | DTYP5 |
| 0x0020 | APPLRH | 0x004D |  | 0x007A |  | 0x00A7 |  | 0x00D4 | DVID6 |
| 0x0021 | APPLRL | 0x004E |  | 0x007B | LRGWE | 0x00A8 |  | 0x00D5 | DTXH6 |
| 0x0022 | LANDN | 0x004F |  | 0x007C |  | 0x00A9 |  | 0x00D6 | DTXL6 |
| 0x0023 |  | 0x0050 | PCKMH | 0x007D | LRGHE | 0x00AA |  | 0x00D7 | DRXH6 |
| 0x0024 |  | 0x0051 | PCKML | 0x007E |  | 0x00AB |  | 0x00D8 | DRXL6 |
| 0x0025 |  | 0x0052 |  | 0x007F |  | 0x00AC |  | 0x00D9 | DTYP6 |
| 0x0026 |  | 0x0053 |  | 0x0080 | PASSW | 0x00AD |  | 0x00DA | DVID7 |
| 0x0027 |  | 0x0054 |  | 0x0081 |  | 0x00AE |  | 0x00DB | DTXH7 |
| 0x0028 |  | 0x0055 |  | 0x0082 |  | 0x00AF |  | 0x00DC | DTXL7 |
| 0x0029 |  | 0x0056 |  | 0x0083 |  | 0x00B0 | DVID0 | 0x00DD | DRXH7 |
| 0x002A |  | 0x0057 |  | 0x0084 |  | 0x00B1 | DTXH0 | 0x00DE | DRXL7 |
| 0x002B |  | 0x0058 | LGADU | 0x0085 |  | 0x00B2 | DTXL0 | 0x00DF | DTYP7 |
| 0x002C |  | 0x0059 | LGADH | 0x0086 |  | 0x00B3 | DRXH0 | 0X00E0 | DVID8 |

|  | Register |  | Register |  | Register |  | Register |  | Register |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x00E1 | DTXH8 | 0x010E | DRXL15 | 0x013B | DTXH23 | 0x0168 | DRXL30 | 0x0195 | DTXH38 |
| 0x00E2 | DTXL8 | 0x010F | DTYP15 | 0x013C | DTXL23 | 0x0169 | DTYP30 | 0x0196 | DTXL38 |
| 0x00E3 | DRXH8 | 0x0110 | DVID16 | 0x013D | DRXH23 | 0x016A | DVID31 | 0x0197 | DRXH38 |
| 0x00E4 | DRXL8 | 0x0111 | DTXH16 | 0x013E | DRXL23 | 0x016B | DTXH31 | 0x0198 | DRXL38 |
| 0x00E5 | DTYP8 | 0x0112 | DTXL16 | 0x013F | DTYP23 | 0x016C | DTXL31 | 0x0199 | DTYP38 |
| 0x00E6 | DVID9 | 0x0113 | DRXH16 | 0x0140 | DVID24 | 0x016D | DRXH31 | 0x019A | DVID39 |
| 0x00E7 | DTXH9 | 0x0114 | DRXL16 | 0x0141 | DTXH24 | 0x016E | DRXL31 | 0x019B | DTXH39 |
| 0x00E8 | DTXL9 | 0x0115 | DTYP16 | 0x0142 | DTXL24 | 0x016F | DTYP31 | 0x019C | DTXL39 |
| 0x00E9 | DRXH9 | 0x0116 | DVID17 | 0x0143 | DRXH24 | 0x0170 | DVID32 | 0x019D | DRXH39 |
| 0x00EA | DRXL9 | 0x0117 | DTXH17 | 0x0144 | DRXL24 | 0x0171 | DTXH32 | 0x019E | DRXL39 |
| 0x00EB | DTYP9 | 0x0118 | DTXL17 | 0x0145 | DTYP24 | 0x0172 | DTXL32 | 0x019F | DTYP39 |
| 0x00EC | DVID10 | 0x0119 | DRXH17 | 0x0146 | DVID25 | 0x0173 | DRXH32 | 0x01A0 |  |
| 0x00ED | DTXH10 | 0x011A | DRXL17 | 0x0147 | DTXH25 | 0x0174 | DRXL32 | 0x01A1 |  |
| 0x00EE | DTXL10 | 0x011B | DTYP17 | 0x0148 | DTXL25 | 0x0175 | DTYP32 | 0x01A2 |  |
| 0x00EF | DRXH10 | 0x011C | DVID18 | 0x0149 | DRXH25 | 0x0176 | DVID33 | 0x01A3 |  |
| 0x00F0 | DRXL10 | 0x011D | DTXH18 | 0x014A | DRXL25 | 0x0177 | DTXH33 | 0x01A4 |  |
| 0x00F1 | DTYP10 | 0x011E | DTXL18 | 0x014B | DTYP25 | 0x0178 | DTXL33 | 0x01A5 |  |
| 0x00F2 | DVID11 | 0x011F | DRXH18 | 0x014C | DVID26 | 0x0179 | DRXH33 | 0x01A6 |  |
| 0x00F3 | DTXH11 | 0x0120 | DRXL18 | 0x014D | DTXH26 | 0x017A | DRXL33 | 0x01A7 |  |
| 0x00F4 | DTXL11 | 0x0121 | DTYP18 | 0x014E | DTXL26 | 0x017B | DTYP33 | 0x01A8 |  |
| 0x00F5 | DRXH11 | 0x0122 | DVID19 | 0x014F | DRXH26 | 0x017C | DVID34 | 0x01A9 |  |
| 0x00F6 | DRXL11 | 0x0123 | DTXH19 | 0x0150 | DRXL26 | 0x017D | DTXH34 | 0x01AA |  |
| 0x00F7 | DTYP11 | 0x0124 | DTXL19 | 0x0151 | DTYP26 | 0x017E | DTXL34 | 0x01AB |  |
| 0x00F8 | DVID12 | 0x0125 | DRXH19 | 0x0152 | DVID27 | 0x017F | DRXH34 | 0x01AC |  |
| 0x00F9 | DTXH12 | 0x0126 | DRXL19 | 0x0153 | DTXH27 | 0x0180 | DRXL34 | 0x01AD |  |
| 0x00FA | DTXL12 | 0x0127 | DTYP19 | 0x0154 | DTXL27 | 0x0181 | DTYP34 | 0x01AE |  |
| 0x00FB | DRXH12 | 0x0128 | DVID20 | 0x0155 | DRXH27 | 0x0182 | DVID35 | 0x01AF |  |
| 0x00FC | DRXL12 | 0x0129 | DTXH20 | 0x0156 | DRXL27 | 0x0183 | DTXH35 | 0x01B0 |  |
| 0x00FD | DTYP12 | 0x012A | DTXL20 | 0x0157 | DTYP27 | 0x0184 | DTXL35 | 0x01B1 |  |
| 0x00FE | DVID13 | 0x012B | DRXH20 | 0x0158 | DVID28 | 0x0185 | DRXH35 | 0x01B2 |  |
| 0x00FF | DTXH13 | 0x012C | DRXL20 | 0x0159 | DTXH28 | 0x0186 | DRXL35 | 0x01B3 |  |
| 0x0100 | DTXL13 | 0x012D | DTYP20 | 0x015A | DTXL28 | 0x0187 | DTYP35 | 0x01B4 |  |
| 0x0101 | DRXH13 | 0x012E | DVID21 | 0x015B | DRXH28 | 0x0188 | DVID36 | 0x01B5 |  |
| 0x0102 | DRXL13 | 0x012F | DTXH21 | 0x015C | DRXL28 | 0x0189 | DTXH36 | 0x01B6 |  |
| 0x0103 | DTYP13 | 0x0130 | DTXL21 | 0x015D | DTYP28 | 0x018A | DTXL36 | 0x01B7 |  |
| 0x0104 | DVID14 | 0x0131 | DRXH21 | 0x015E | DVID29 | 0x018B | DRXH36 | 0x01B8 |  |
| 0x0105 | DTXH14 | 0x0132 | DRXL21 | 0x015F | DTXH29 | 0x018C | DRXL36 | 0x01B9 |  |
| 0x0106 | DTXL14 | 0x0133 | DTYP21 | 0x0160 | DTXL29 | 0x018D | DTYP36 | 0x01BA |  |
| 0x0107 | DRXH14 | 0x0134 | DVID22 | 0x0161 | DRXH29 | 0x018E | DVID37 | 0x01BB |  |
| 0x0108 | DRXL14 | 0x0135 | DTXH22 | 0x0162 | DRXL29 | 0x018F | DTXH37 | 0x01BC |  |
| 0x0109 | DTYP14 | 0x0136 | DTXL22 | 0x0163 | DTYP29 | 0x0190 | DTXL37 | 0x01BD |  |
| 0x010A | DVID15 | 0x0137 | DRXH22 | 0x0164 | DVID30 | 0x0191 | DRXH37 | 0x01BE |  |
| 0x010B | DTXH15 | 0x0138 | DRXL22 | 0x0165 | DTXH30 | 0x0192 | DRXL37 | 0x01BF |  |
| 0x010C | DTXL15 | 0x0139 | DTYP22 | 0x0166 | DTXL30 | 0x0193 | DTYP37 | 0x01C0 |  |
| 0x010D | DRXH15 | 0x013A | DVID23 | 0x0167 | DRXH30 | 0x0194 | DVID38 | 0x01C1 |  |

| Reg: SYMSET (System Setup) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  |  |  |  |  |  |  | SYSINIT |

| Bit | Name | Description |
| --- | --- | --- |
| 7 |  |  |
| 6 |  |  |
| 5 |  |  |
| 4 |  |  |
| 3 |  |  |
| 2 |  |  |
| 1 |  |  |
| 0 | SYSINIT | 1: System has been initialized. 0: System has not been initialized. |

| Reg: SYSSRC (System Input Source) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  |  |  |  |  | GPSSRC | SPDSRC |  |

| Bit | Name | Description |
| --- | --- | --- |
| 7 |  |  |
| 6 |  |  |
| 5 |  |  |
| 4 |  |  |
| 3 |  |  |
| 2 | GPSSRC | 1: The source for GPS data is the RS232 line 0: The source for GPS data is the GPS module |
| 1-0 | SPDSRC | 11: Speed source is GPS 10: Speed source is the speed pickup 01: Speed source is Simulated 00: Speed source is Ranked from GPS:pickup to Simulated depending on errors |

| Register | Name |
| --- | --- |
| TNKMH | Tank Level Major Warning High Byte |
| TNKML | Tank Level Major Warning Low Byte |
| TNKAH | Tank Level Minor Warning High Byte |
| TNKAL | Tank Level Minor Warning Low Byte |
| TNKCH | Tank Capacity High Byte |
| TNKCL | Tank Capacity Low Byte |
| APPLRH | Default Application Rate High Byte |
| APPLRL | Default Application Rate Low Byte |
| ACTRH | Active Row Range Upper Limit |
| ACTRL | Active Row Range Lower Limit |
| LANDN | Current Land Number |
| ROWS | Number of Rows on Implement |
| RWWD | Implement Row Width |
| SIMSPD | Simulated Speed |
| IMPLID | Implement ID (10 bytes) |
| IMPLTP | Implement Type (2 bytes) |
| FERTTP | Fertiliser Type (10 bytes) |
| FRTDH | Fertiliser Density High Byte |
| FRTDL | Fertiliser Density Low Byte |
| SEEDTP | Seed Type (10 bytes) |
| PCKMH | Pickup mm Per Pulse High Byte |
| PCKML | Pickup mm Per Pulse Low Byte |
| PCKSP | Speed Pickup Row Identifier |
| PCKPR | Pressure Pickup Row Identifier |
| PCKPX | Proximity Pickup Row Identifier |
| LGADU | Log Pointer Flash Address Upper Byte |
| LGADH | Log Pointer Flash Address High Byte |
| LGADL | Log Pointer Flash Address Low Byte |
| SERLH | Serial Number High Byte |
| SERLL | Serial Number Low Byte |
| TMZN | Time Zone |
| LRDHM | Last Reset Daily Hours And Minutes |
| LRDY | Last Reset Daily Years |
| LRDMD | Last Reset Daily Month and Day |
| LRDHW | Last Reset Daily Hours Worked |
| LRDVO | Last Reset Daily Volume Applied (L) |
| LRDWE | Last Reset Daily Weight Applied (kg) |
| LRDHE | Last Reset Daily Hectares Planted (hec) |
| LRGHM | Last Reset Grand Hours And Minutes |
| LRGY | Last Reset Grand Years |
| LRGMD | Last Reset Grand Month and Day |
| LRGHW | Last Reset Grand Hours Worked |
| LRGVO | Last Reset Grand Volume Applied (L) |
| LRGWE | Last Reset Grand Weight Applied (kg) |
| LRGHE | Last Reset Grand Hectares Planted (hec) |
| PASSW | Password (10 Nibbles) |
| RGDEV | Number Of Registered Devices |
| FDEV | Number of registered flow control devices |
| PDEV | Number of registered peripheral devices |
| ENVCH | Envelope Counter High Byte |
| ENVCL | Envelope Counter Low Byte |
| DVIDx | Device ID Code |
| DRXHx | Device Receive Envelope High Byte |
| DRXLx | Device Receive Envelope Low Byte |
| DTXHx | Device Transmit Envelope High Byte |
| DTXLx | Device Transmit Envelope Low Byte |
| DTYPx | Device Type |
| GDD0 | Record Date Day 0 |
| GDD1 | Record Date Day 1 |
| GDM0 | Record Date Month 0 |
| GDM1 | Record Date Month 1 |
| GDY0 | Record Date Year 0 |
| GDY1 | Record Date Year 1 |

# Flash Memory Configuration

| 0x0000 |  |
| --- | --- |
|  | Log Data memory |
| 0x6000 |  |
|  | Shape Files |
| 0x7FFF |  |